GNG2101

Design Project Progress Update

HydroHold AND F2.1

Submitted by:

Kaitlynn Sampel, 300200475

Leena Hafez, 300298840

Bryn Robins, 300281442

Mitchell Ryan Bromberg, 300380559

Wency Palma, 300282037

February 7, 2025

University of Ottawa

Table of Contents

Table of Contents	ii

List of Figures	iv

List of Tables	v

Glossary	vii

1.0 Introduction	1

2.0 Sustainability Report	2

2.1 Economic Impact	3

2.2 Social Impact	5

2.3 Environmental Impact	8

3.0 LCA Report	10

4.0 Design for X	17

4.1 Design for Repairability	17

4.2 Design for Manufacturing	20

4.3 Design for Ease of Use	21

4.4 Design for Assembly	23

4.5 Design for Cost	25

5.0 Client Need Statement Generation	26

6.0 Problem Statement	32

7.0 Metrics & Benchmarking	33

8.0 Development of Target Specifications	40

9.0 Concept Development Process	46

9.1 Definition of Product Subsystems	46

9.2 Global Concept Generation	50

9.3 Justification of Global Concept Solution, and Concept Revision	56

9.4 Relation of Global Concept Selection to Target Specifications & DFX Modalities	59

10.0 Detailed Design and Design Refinement	59

10.1 Client Feedback Summary	60

10.2 Detailed Design Concept	61

10.3 Design Considerations Regarding Fulfillment of DFX Factors	65

10.4 Relevant Skills and Resources to be Used	67

10.5 Project Timeline	68

10.6 Critical Assumptions	71

10.7 Product BOM	73

11.0 Project Plan Update	76

12.0 Conclusions	78

Bibliography	80

List of Figures

Figure 1: Triple Bottom Line Diagram	10

Figure 2: Diagram of Cup holder Design System	11

Figure 3: Kalolary Cup Holder	35

Figure 4: Steady Sip Cup Holder	36

Figure 5: Cup Cruiser Crew Cup Holder	37

Figure 6: Free Body Diagram of Cup	41

Figure 7: Desk Lamp with Segmented Arm	48

Figure 8: Desk Lamp with Hinged Arm	49

Figure 9: First Global Concept Sketch	51

Figure 10: Second Global Concept	52

Figure 11: Third Global Concept Sketch	53

Figure 12: Fourth Global Concept Sketch	54

Figure 13: Selected Global Concept - First Variant	57

Figure 14: Selected Global Concept - Second Variant	58

Figure 15: Selected Global Concept - Third Variant	58

Figure 16: Cup Holder Detailed Drawing	62

Figure 17: C-Clamp Base Detailed Drawing	63

Figure 18: Clamp Sub-Assembly	64

Figure 19: Assembly Drawing of Final Proposed Design	65

Figure 20: Project Plan (Gantt Chart) Update	77

List of Tables

Table 1: Glossary	vii

Table 2: Impact of Acquiring Raw Material	12

Table 3: Impact of Material Processing	12

Table 4: Impact of Component Manufacturing	13

Table 5: Impact of Product Packaging	14

Table 6: Impact of Product Transportation	14

Table 7: Impact of Product Effective Lifespan	15

Table 8: Impact of Product and Product Component Design	15

Table 9: Overall Environmental Impact of Product	16

Table 10: Revised Overall Environmental Impact of Product	17

Table 11: Translation of Client Statements Into Need Statements	26

Table 12: Ranking of Need Statement Importance with Justification	28

Table 13: Metrics Associated with Client Needs & Need Statements	33

Table 14: Benchmarked Products (Description & Source)	35

Table 15: Decision Matrix for Benchmarked Products	38

Table 16: Target Specifications for Binary Metrics	44

Table 17: Target Specifications Associated with Metrics	45

Table 18: Potential Design Solutions to Proposed Product Subsystems	47

Table 19: Decision Matrix for Global Concept Evaluation & Selection	54

Table 20: Implementation Plan	68

Table 21: Detailed BOM for Product	74

Glossary

Table 1: Glossary

| Term | Acronym | Definition |
| --- | --- | --- |
| Design For X | DFX | Design of a product or service to fulfill a specific need as outlined by international established standards and methods. |
| Life Cycle Analysis | LCA | A process by which a specific portion of a product’s life can be scrutinized to quantify impact. |
| Triple Bottom Line | TBL | A framework that evaluates organizational or project performance based on three interconnected dimensions (economic, social, and environmental). |
| Computer Aided Design | CAD | The use of computer software to facilitate the design process. |
| Bill of Materials | BOM | A detailed list of all elements that are required to complete a product, including their cost, quantity, and other factors/characteristics. Typically expressed in a tabular manner. |

# 1.0 Introduction

The goal of the subsequently analyzed project is to design and build a cup holder to attach to the tray of a wheelchair, as requested by the client (Travis &/on behalf of Nicky). The following document will include a preliminary sustainability report, which details the positive and negative impacts of the product with respect to several key sustainability modalities, namely, economic impact, social impact, and environmental impact. There are many types of sustainability: environmental, economic, social, the lifecycle of the product, etc. To evaluate the sustainability the triple bottom line (TBL) framework and a Life Cycle Assessment framework (LCA) will be used. These analysis strategies and frameworks will be examined in far greater detail in subsequent sections of the report.

To complement the sustainability analysis, various DFX principles will be examined to determine whether they would complement the design process of the product. DFX (which stands for “Design For X”) is a design tool to help designers focus on related thematic elements. Keeping in mind the client’s statements and the potential constraints and pitfalls of the project, relevant DFXs were selected for analysis, and potential implementation, as detailed below. Ultimately, the modalities of design for repairability, design for manufacturing, design for ease of use, design for assembly, and design to cost were determined to be the most pertinent. Each of these DFX strategies are detailed below, including descriptions of their use, typical metrics to measure their success, potential constraints, potential applicable design criteria, and a brief commentary on methodologies for applying the DFX to the product and its design process.

In addition to the sustainability report, LCA, and preliminary DFX analysis, the document also addresses the first steps in the engineering problem refinement process. Namely, client needs are listed, transformed into corresponding and appropriate need statements, and ranked in terms of their perceived importance based on customer/client feedback, and team consensus. Metrics which may be used to quantifiably measure the level of adherence of a given design solution to the client’s needs will be generated. Benchmarking of similar products will also be performed to determine existing design solutions and their strengths and shortcomings pertaining to fulfilling the particular needs of the client. Premised on the aforementioned steps, target specifications for each metric will be created as a means of evaluating the generated design solutions.

Having evaluated the various proposed design solutions, a select few were proposed to the client in the next client meeting. Based on the client’s feedback, it was possible to develop a global detailed design, carefully plan out the implementation of said detailed design, and create a detailed BOM to plan out the economic factors associated with the creation of the product.

# 2.0 Sustainability Report

Part of the aim of this particular project is to instill sustainable design principles in the finished product. To this end, an analysis of the impact of the finished product is warranted. This higher-level analysis can later be refined as more details regarding the final product and generated solution become clear. At this stage, only a rather superficial analysis will likely be possible, though this will serve as a building block for later refinement of sustainability goals and overall product improvement and iteration.

A three-pronged sustainability analysis will ensue, analyzing the positive and negative economic, social, and environmental impact of the designed product. While each of these modalities are, in theory, distinct, it is important to note that there may be overlap in their analysis. What’s more, owing to the aforementioned links between the modalities, trade-offs may exist in applying sustainability principles related to each sustainability aspect. This is to say that positive economic impact for instance, may come at the cost of reduced social impact.

## 2.1 Economic Impact

Economic impact is typically the most readily defined, and easily quantified sustainability modality. An economically sustainable design, in short, makes money. The profit margin of the product or service is sufficiently healthy that it can be in the market for an extended period of time.

Seeing as the product has yet to be designed, and as such yet to be manufactured, quantifying the economic impact of the cup holder is uniquely challenging. This being said, some crude estimations may be used. For instance, in the client interview, it became very apparent that the client wishes to have a 3D printed product (at the very least in part). Thus, one can estimate the economic impact of using 3D printing to design and manufacture our product, compared to traditional methods for making cupholders.

On the positive side, the use of 3D printing will greatly reduce the manufacturing costs associated with the production of the cupholder when compared to traditional manufacturing methods like injection molding. The injection molding process requires custom molds, which are known for being both costly and time consuming to make; 3D printing relies on digital design files as opposed to molds, making it much more cost-effective. Methods like injection molding also have greater costs associated with machinery and maintenance [4]. As such, for low-volume production, the use of 3D printing will increase the profitability of the product, classifying it as a positive economic impact. Additionally, 3D printing makes manufacturing more efficient by automating many stages of the process. This will significantly reduce the need for manual labour, which will in turn lower labour costs and increase the overall profit margin for the product. Finally, 3D printed products are made layer by layer. This process creates a much more lightweight product as opposed to their traditionally made counterparts (i.e., products made via injection molding). For the product, this reduction in weight directly translates to lower shipping fees, as they are typically based on the size and weight to be shipped. 3D printing also allows for more localized manufacturing, which further reduces shipping fees; instead of paying more to send the products around the world, they can be manufactured in regions of higher demand to be sent out locally.

On the negative side, 3D printing is only profitable for low-volume production. Since this process requires continuous material setup, loading and monitoring, the time to produce one singular cupholder is much higher than when using other mass manufacturing methods. Additionally, when considering high-volume production, the cost of materials such as ABS and PLA can add up to be much more expensive than the bulk materials that would be used for injection molding. So, in the way of mass production, the use of a 3D printer would be less cost-effective than the other traditional manufacturing methods. 3D printers also have a much higher cost of maintenance for long-term projects. There are several moving parts in each printer, including belts, motors and print heads, which may wear down over time (or get clogged, in the case of print heads) and can be costly to repair and/or replace. 3D printing is also not a perfect process. Print failures, whether due to incorrect settings or materials, or due to mechanical malfunction, can be costly for low-volume production levels. The lost material, time to reset the machine, and any replacements required can add up and may create a negative economic impact. Finally, this product was designed primarily for patients with high level cerebral palsy and other individuals with mobility impairments. One of the economic drawbacks of designing a product for a specific group of people is the limited market size. Since the product is intended for a smaller demographic, there will inherently be less overall demand for the product, which decreases the income potential. So, while the product may be of great value to our intended audience, our product will not achieve a widespread level of profitability that other, more generalized cupholders may achieve.

## 2.2 Social Impact

The designed product is likely to be focussed on social impact, with greater contributions to this area than to the other modalities of sustainability. Social sustainability is a rather more complex examination of a product’s impact, as compared to the other sustainability modalities. While the concept of social sustainability is not particularly concretely defined, many argue that it represents the social inclusion of all individuals, with the ultimate goal of a greater collective, social well-being [1].

In this respect, the design of a cup for a person with a disability counts many aspects of social sustainability in that it promotes social inclusion. Thus, the positive impacts include promoting access to specific products and product classes which may not have been accessible prior. This could be construed as resulting in an improved quality of life, improved ability to be independent or cost-effective solutions. All of the above represent positive contributions to collective social well-being, and indicate strong, positive implementation of social sustainability principles.

A major advantage of a wheelchair tray cup holder is how it boosts user independence. The tray securely holding a cup removes the need for “external” assistance when carrying or drinking from a beverage. This enables users to complete essential tasks without relying on others. Having this cup holder can greatly boost user’s independence and self-reliance, leading to greater confidence and mental well-being. Being able to handle small tasks on their own promotes a sense of dignity and freedom, while also reducing the psychological and emotional impact of reliance on caregivers [5].

Simple conveniences, like being able to safely carry a cup while in a wheelchair, can greatly enhance a user’s overall quality of life. The ability to enjoy daily activities, such as drinking water or milk, without constant assistance, helps users feel more integrated into social situations. The design of the cup holder not only offers a practical solution to a common challenge faced by wheelchair users but also contributes to emotional well-being by making users feel more normal and connected to society. This small innovation helps reduce feelings of isolation, encourages social interaction, and supports greater participation in both public and social environments.

3D-printing the cup holder offers a cost-effective solution for wheelchair users, providing high functionality at an affordable price. By focusing on simplicity and practicality, the design addresses a common need without the financial strain of more expensive, specialized mobility aids. This cost-effective approach ensures that the product is accessible to a wider range of individuals, regardless of their socioeconomic background. If priced reasonably, it could benefit people in lower-income areas or countries with limited access to disability aids.

For their part, negative social impacts are markedly more difficult to find. Inherent in the product is the idea of promoting social well-being and sustainability. As such, it is premised on expressly reducing negative social sustainability impacts.

While the cup holder does enhance convenience, it could also lead to an increased reliance on assistive devices. This device may reinforce the notion that individuals with mobility challenges need constant adjustments and modifications to their environment, rather than prioritizing the goal of enhancing their overall independence [6].

The presence of additional assistive devices, such as a wheelchair cup holder, can inadvertently draw attention to the user’s disability, despite its intention to provide convenience and comfort. The visibility of assistive devices may lead to social stigma and negative stereotyping, with the cup holder serving as a constant reminder of the disability. For some users, this could result in feelings of embarrassment or discomfort, especially if the device is highly noticeable. The stigma surrounding disability-related tools can contribute to social isolation or discrimination, particularly in societies where disability awareness and acceptance are limited [7].

While 3D printing has the potential to provide affordable, custom-made devices like cup holders, this technology is not widely accessible in low-income or developing regions. The high costs of materials and reprinting damaged parts make it difficult for individuals in these areas to afford or maintain 3D-printed products, limiting their opportunities to enhance their independence and quality of life [8].

In conclusion, the wheelchair tray cup holder can enhance independence and quality of life for users, but it also carries risks like reinforcing reliance on assistive devices and increasing social stigma. By prioritizing accessibility, the cup holder can maximize its positive impact and help enhance the daily lives of wheelchair users.

## 2.3 Environmental Impact

Achieving an overall positive environmental impact may be the most challenging aspect of sustainability to implement. Seeing as the project is to be developed under stringent budget constraints, and with practical limitations on access to design and manufacturing technologies, reduction of harmful environmental impact may not be possible in certain respects. While it would be ideal to design a product that has a net positive impact on the environment, it might not be entirely possible. One of the positive environmental impacts related to our design is that the material used to 3D print the cupholder (i.e., PLA) is biodegradable and can be reprocessed/recycled at the end of its lifecycle as a means to reduce its negative environmental impact, especially when compared to traditionally manufactured plastics, which often create waste/pollution [9]. Additionally, as mentioned in Section 2.1, 3D printing significantly reduces the weight of each cupholder. With less weight, the vehicles transporting the products would need to consume less fuel, which directly lowers CO2 emissions related to shipping and transportation.  3D printing, as aforementioned, also allows for more localized manufacturing. Printing the products in areas of high demand would significantly reduce CO2 emissions that would otherwise result from long-distance transportation and shipping.

While 3D printing the product may have its benefits, there are still many negative environmental impacts that result from the process. First, sourcing the filament (PLA or ABS) is an energy-intensive process that may have degenerative effects on the environment. PLA is a bioplastic typically made from either corn or sugarcane. The process of harvesting these crops, however, can contribute to water pollution, deforestation and soil degradation [10]. Once this material is sourced and placed in the 3D printer, it gets melted so that it can be used to create the cupholder. Applying heat to these plastics releases chemicals which may be harmful to both the people maintaining the machines, and the environment. Finally, despite the advantages of 3D printing the cupholders, this is still a process that generates waste. While plastics like PLA are considered biodegradable, they can still contribute to the waste that ends up in landfills if not disposed of properly. Incorrect disposal may lead to microplastics entering the environment, which can cause many long-term problems.

Figure 1: Triple Bottom Line Diagram

# 3.0 LCA Report

The following Life Cycle Analysis (LCA) for a portable, clampable cup holder for affixation to surfaces. It is thought that this product may align with the vision for the cup holder to be developed, as outlined earlier in the document. The LCA aims to provide a succinct, but detailed representation of the potential environmental impacts of the cup to be designed, through understanding the impact of a similar or roughly equivalent product.

The analysis is aimed at understanding the ecological impact of the product's entire lifespan, from creation, through its lifetime, until end of life and ultimate disposal. Hence, the analysis is a cradle-to-grave analysis. The length of the product’s lifespan is difficult to say, seeing as life cycle statistics are not readily available, nor is life cycle testing possible as a means of gathering more data. As a consequence, a rough estimate of a five-year lifespan will be used.

The cup holder system over its lifespan can be represented by the diagram in the figure below (Figure 2). Note that this diagram likely constitutes an oversimplification of the product life cycle system. With more specific insight into the product creation methodology/system, a more detailed analysis and system diagram could be created.

Figure 2: Diagram of Cup holder Design System

Seeing as the frame of the material is made principally made of aluminum, with some rubber components and padding, and some plastic elements, the only necessary materials to acquire are those mentioned prior. The results of this raw material impact analysis following step 1 of the framework provided on the GNG 2101 Brightspace are shown in the below table (Table 3).

Table 2: Impact of Acquiring Raw Material

| Type of Raw Material | Points |
| --- | --- |
| Aluminum | 1 |
| Rubber | 1 |
| Plastic | 1 |
| Total | 3 |

With respect to all of the above materials, they would all need to be processed and formed into their final shape. Pursuant to the process outlined on the course Brightspace, points can be attributed to all of the above materials, as they would all need to be processed.

Table 3: Impact of Material Processing

| Plastics and metals in the product | Impact |
| --- | --- |
| Aluminum | 1 |
| Plastic | 1 |
| Rubber | 1 |
| Total | 3 |

The following table represents the assessment of the manufacturing impact associated with the creation of the product. Each manufactured component is awarded a point in terms of ecological impact, as outlined in the proposed LCA framework.

Table 4: Impact of Component Manufacturing

| Different parts and pieces | Impact |
| --- | --- |
| Clamp screw | 1 |
| Clamp screw end cap | 1 |
| Cup receptacle | 1 |
| Inferior metal bodywork | 1 |
| Superior metal bodywork | 1 |
| Total Points | 5 |

Considerations also have to made regarding the packaging of the finished product. Per the LCA framework, the following impact can be associated with the product’s life cycle. Seeing as the product is not owned or purchased, plastic and cardboard packaging will be assumed, seeing as this kind of packaging is relatively standard, particularly in the case of shipped products.

Table 5: Impact of Product Packaging

| Packaging | Impact |
| --- | --- |
| Plastic and cardboard packaging | 10 |
| Total Points | 10 |

Whether it be transportation to store fronts, or transportation to customers who purchased the product on an online storefront, considerations must be made with respect to the environmental impact of shipping the product. Pursuant to the framework, the impact of this transportation can be expressed as shown in the below table (Table 7).

Table 6: Impact of Product Transportation

| Transport | Impact |
| --- | --- |
| By plane, truck or boat | 15 |
| Total | 15 |

As mentioned prior, quantifying the life cycle of the examined product is extremely difficult given the circumstances. This being said, in being consistent with that which was assumed earlier (premised on some very crude, but logical assumptions and observations) a lifespan of five years will, once again, be assumed. The resultant impact of this length of lifespan is summarized in Table 8.

Table 7: Impact of Product Effective Lifespan

| Product Lifespan | Impact |
| --- | --- |
| 5 Years | 10 |
| Total | 10 |

Finally, considerations must be made with regard to the environmental impact of the product’s disposal. Seeing as aluminum (depending on treatment and other material properties), the ecological impact of the product’s disposal may be reduced in comparison to those products which must be entirely disposed of. Table 9, presented below, indicates the quantifiable ecological impact of the product’s disposal.

Table 8: Impact of Product and Product Component Design

| Disposal Methodology | Impact |
| --- | --- |
| Parts of the product may be recycled | 5 |
| Total | 5 |

Having, relatively exhaustively, quantified the environmental impact associated with the lifespan of the cup holder product, the overall impact can now be determined. Through summing the overall contributions of the various aspects of the products manufacturing, use, and overall life, the overall quantified impact can be computed. The overall impact figure is shown below in Table 10.

Table 9: Overall Environmental Impact of Product

| Total Points | 51 |
| --- | --- |

While the above figure is certainly not exorbitant it could certainly be improved upon. In order to reduce the environmental impact of the cup holder, there are a few actions that could be taken. The most foremost among these actions is the use of more recyclable materials. The exclusive use of metals and/or plastics that can be recycled would greatly reduce the overall impact of the product. What’s more, careful selection of product packaging, say the sole use of carboard, could also go a long way in reducing the environmental impact of the product.

While it may seem easy to reduce the impact of the product pursuant to the above observations, it is important to keep in mind that there are certain areas in which environmental impact reduction is not feasible. For instance, being a consumer-oriented good, it simply isn’t feasible to eliminate the need for transportation of the cup holder. While transportation could perhaps be approached in a more thoughtful, and less impactful way, it cannot be eliminated altogether as the product will need to be shipped to storefronts and/or customers.

In re-evaluating the environmental impact of the cup holder after making the adjustments outlined below, the total impact due to disposal falls from 5 to 0, and the total impact due to packaging falls from 10 to 5. Hence, the overall product impact is reduced by 10 points, falling to 41 points. This is shown in Table 11, below.

Table 10: Revised Overall Environmental Impact of Product

| Total Points | 41 |
| --- | --- |

Pursuant to the above LCA, it is evident that aspects of the cup holder to be created for the client such as packaging, and manufacturing should be considered so as to reduce the overall impact of the product.

# 4.0 Design for X

Through both discussions with the client, and identification of product constraints, five unique DFXs became apparent as being more pertinent than others. These include design for repairability, design for manufacturability, design for ease of use, design for assembly, and design for cost.

## 4.1 Design for Repairability

During the client meeting, which was analyzed in depth in Deliverable C, it was mentioned many times by the client that they liked the idea of a 3D printed product. They explained that ComputerWise, the client’s base of operations, has a number of 3D printers themselves. Thus, any 3D printed cup could be replaced, or repaired by simply 3D printing a new one using the CAD files developed for the prototype in future.

This modality of DFX has gained extensive traction in the electronics sector. Much of the information, and the design principles applicable to the domain of design for repairability are thus in relation to the design and production of electronics and ancillary/related goods. This being said, a number of the outlined principles are still applicable to the design of physical goods, such as the cup holder to be designed for the current client.

The fundamental purpose of design for repairability is the concept of making it as easy as possible for consumers to repair their own products.

Principles associated with this DFX modality which are relevant beyond the realm of electronics repairability include [2]:

- Use of a modular design to allow easy assembly and disassembly, including removal and replacement of defective components

- The use of magnets or fasteners instead of adhesives wherever possible

- The use of common fasteners that are readily available

- Inclusion of a product instruction manual/overview which explains the replacement process for critical product components

- The use of easily replaceable and readily available components sourced from the design company or a reliable third party

- The design of well thought-out and desirable products that consumers want to use and continually repair

Metrics premised on the above are rather difficult to find, seeing as most of the above principles are Boolean or binary in nature – they either are fulfilled, or they aren’t. Metrics such as the following would represent excellent, quantifiable measurements of one’s success in adhering to design for repairability principles [2]:

- Exclusive use of standard fasteners in accordance with standards and regulations as outlined by reputable organizations (ASTM, SAE or other)

- Development of a complete and detailed product manual

- Average cost of repair for a given component

Sample constraints for design for repairability would principally relate to product functionality. In essence, the repairability of a component or product is practically limited by the necessity that the component or product performs is prescribed function. For instance, in the case of this particular project, previous prototypes included machined metal clamp sections. Access to machine shops is typically limited for the general public, hence the fixation mechanism of the cup holder largely does not comply with design for repairability principles. This being said, the connection mechanism must be robust, which may not be achievable with a different mechanism making use of more repairability-oriented processes or materials. As such, the degree of repairability of the finished product is limited by the function that its constituent components must fulfill.

Other factors such as cost, could also represent constraints in implementing design for repairability principles. For instance, the implementation of a modular design, while ideal, means designing, and/or purchasing many fasteners to permit assembly. This could come at an exorbitant cost, or require extensive time and effort, which may not be justifiable in certain cases.

Sample design criteria related to design for repairability include limiting the number of separate parts, allowing for easier repair, as well as making the way the cupholder attaches to the wheelchair non-permanent so it can be removed and more easily fixed.

## 4.2 Design for Manufacturing

Designing for manufacturing requires that the final design be easy to produce, while maintaining precision and accuracy. Every component of the design must align with this principle. The four key factors (or criteria) guiding the design process are environmental sustainability, simplified part design, durability, and cost-effectiveness.

Some constraints related to this DFX modality are that the client prefers the cup holder to be 3D printed. This severely limits options with respect to the manufacturing processes utilized. The client also prefers the colour of the cup holder to be red, limiting the 3D filament that can be used to only kinds that can be made red or that can be painted red after printing the cup holder.

To ensure the product is environmentally sustainable, the manufacturing process should minimize the ecological impact of its materials and production processes. This includes researching and selecting biodegradable or recyclable materials, such as eco-friendly 3D printing filaments. Additionally, the manufacturing process should focus on reducing waste and excess material during manufacturing, which will lower production byproducts and enhance sustainability. Careful consideration will also be given to material reduction to ensure it does not compromise the product's structural integrity. By adopting these practices, the product will balance sustainability with durability.

To streamline manufacturing, the design will prioritize simplicity by minimizing the number of moving or separate parts. The goal is to limit the product to no more than two components, ensuring that all parts can be 3D printed without requiring complex support structures.

The manufacturing process will be optimized to enhance the product's structural strength, resilience, and longevity. For example, the manufacturing process will ensure the 3D printing temperature is maintained within 2°C of the material's recommended temperature, which is critical for achieving consistent quality and strength.

To maintain cost-effectiveness, the manufacturing process should focus on reducing printing time and selecting more affordable 3D printing filaments without compromising quality. This approach will help minimize production costs while delivering a high-quality final product.

## 4.3 Design for Ease of Use

Based on the client’s mobility challenges in relation to their disability, implementation of design for ease of use, or design for usability appears to be of paramount importance. Every aspect of the finished product must be carefully and thoughtfully tailored to the specific abilities and needs of the client. While in the case of most clients, a prototype lacking usability may compromise product success to a degree, in the case of the particular client, failure to deliver a usable design could result in the client being altogether unable to use the product in any capacity.

There are relatively few measurable quantities and, thus, metrics that can be used to quantify a products usability. This aspect of a product is highly subjective and unfortunately must likely be measured as such. Strategies for gaining quantitative measures could include conceiving a point system or scale on which users can measure the quality of a given prototype.

While the above may seem like a logical and workable idea, there are issues that exist in so far as the current project. Namely, the mobility challenges of the client are rather unique. As such, impressions from polled “potential clients” may not adequately capture the impressions of the client or an individual having a similar experience or worldview of the client. Consequently, it is imperative that the subsequent client meetings are used as opportunities to evaluate generated prototypes. If such opportunities are not harnessed, the likelihood of creating a failed product will increase drastically.

Regardless, there are general methods of increasing the usability of a design. The requested product is a cup holder first and foremost. Simplifying generated concepts to just that, a cup holder may work in favor towards the usability of any prototypes. However, it is also possible to have usability and functionality simultaneously. Adding new functions to a design does not correlate directly to a decrease in usability. Additional functions may be affixed to the design while maintaining usability, if the function is easy to use and does not impede the other functions. For example, the client mentioned the addition of a straw holder as a potential improvement to the previous prototypes they have received. Positioning the straw holder where it allows for easy access to both the straws and the cup maintains ease of use while having additional functions.

The integrity and quality of the design is also included in designing for ease of use. In the situation where the cup holder is 3D printed with a large nozzle, or the quality is reduced for the sake of reduced manufacturing time, it may lead to a rough interior and exterior surface, effectively making fitting cups in the holder, maneuvering the cup holder, or any additional functions difficult and uncomfortable. Therefore, the adherence of the product to ease of use principles is limited not only by typical constraints (such as time and money), but also by the manufacturing processes and machines available to build the product.

Sample design criteria t create usable designs include simple assembly and disassembly, a small number of constituent components, and the use of icons to communicate product functionality.

## 4.4 Design for Assembly

Designing for assembly requires that the final design be straightforward to assemble without any sacrifice in product functionality and quality. The foundation for product development for assembly requires component minimization, simplifying the assembly process, and enhancing overall durability.

To achieve component minimization, the design must include as few parts as possible, by using a simplified part design to integrate multiple functions into singular components. Reducing the number of unique elements allows the assembly to become less prone to error and hastens the manufacturing process. Combining multiple components into a single 3D printed piece will eliminate the need for additional fasteners. Designing with consideration to this approach allows the assembly to maintain or potentially improve structural integrity while reducing the necessary parts for assembly. This also helps with repairability, with less parts there are less parts that can individually fail, this means when something does break it is easier to locate the broken piece and replace it.

Maintaining durability is equally crucial to the final design, therefore longevity and sturdiness will remain a priority in designing connections between the few components. Nevertheless, to address the impact of increased stresses on a reduced number of components while maintaining ease of assembly, selecting material is critical for prolonged structural integrity. Therefore, strong and lightweight material fitting the design’s objective and adhering to the preferred manufacturing methods of the client, such as a plastic that is 3D printer compatible , as well as the elimination of excess fasteners which is most likely part of products to fail, the product, will be invaluable to integrate into the final design.

To measure how effective this DFX is one can look at metrics such as total assembly time. The time from start to finish shouldn't take a long time. One can also look at if any tools need to be used to assemble it. The assembly process should ideally be limited to only one or less tools to assemble the product.

A satisfactory design outcome can be achieved through the careful application of the key principles that lead to a simplified final design with functionality and durability in mind. This approach will allow the simplified assembly to successfully meet the client’s expectations.

## 4.5 Design for Cost

Design to cost represents the concept of understanding the costs associated with each stage and element of a project or products creation or implementation. It sees use primarily in ensuring healthy economic performance of goods and services in increasingly competition-laden markets [3]. While this is somewhat at odds with the purpose of the product to be designed, the principles to be used in design to cost may be helpful in ensuring compliance with the stringent $50 budget allocated to this project, as well as contributing to other DFX modalities. For instance, prediction, and adherence to budget may improve the repairability of the finished product in ensuring that components used in the products construction can be manufactured or purchased at a reasonable price.

Being concerned with costs (i.e. quantifiable quantities), there are a number of potential metrics which could see use in adhering to design to cost principles. For instance, product manufacturing cost may constitute a pertinent metric, as could cost for replacement for individual constituent components of the product. The latter, could help with design for repairability principles, as outlined above.

Constraints on this design modality are numerous. They could include concepts such as minimum fixed costs. This is to say that certain modalities of the product may put constraints on a budget in that they cannot possibly reduced beyond a certain point. A good example would be material costs. One cannot create a product for less money than the cost of the materials that compose it. In relation to the particular project being undertaken, an exceptionally relevant constraint would be time. A lack of time to complete a given project may reduce opportunities to strictly adhere to budget, given that less thought can be put into design and economics decisions.

Design criteria related to design for cost could include low material costs, low manufacturing costs, or the use of refurbished or recycled materials (which may be cheaper to use)

5.0 Client Need Statement Generation

In order to fully comprehend the scope and stipulations of the problem to be solved, client statements from the preliminary client interview/meeting must be turned into corresponding need statements. As such, the below table (Table 9) shows the translational process from client statement to need statement.

Table 11: Translation of Client Statements Into Need Statements

| Client Statement | Corresponding Need Statement |
| --- | --- |
| Nicky frequently knocks over her cup. | The cup holder prevents the cup within from being knocked over. |
| Nicky always has her cup with her throughout the day. She uses it at home and work. | The cup holder can be used for long, uninterrupted periods of time. The cup holder can be used in a variety of locations. |
| Nicky only typically uses the one cup. | The cup holder works well for the particular model of cup used. |
| The cup holder doesn’t really need to move. Ideally, it should fit on the wheelchair tray in the same position as Nicky’s current cup holder. | The cup holder is fixed in one position. The cup holder is positioned towards the back left corner of the wheelchair tray. |
| Colour isn’t really a big deal. Maybe red would be nice. | The cup holder is red |
| The old design was good, the metal arm was just too long, and the holder wasn’t in the right place. | The cup holder is positioned towards the back left corner of the wheelchair tray. (Reiterated) |
| We’re not too worried about extra features, functionality is the most important. Maybe a straw holder would be good, just an idea. | The cup holder is functional in design. The cup holder has a straw holder. |
| I like the idea of 3D printing. If something breaks, we can just replace it here. | The cup holder is manufactured by 3D printing. |
| I think a clamp is the way to go in terms of fixation. | The cup holder is attached to the wheelchair tray via a clamp. |
| I’d rather not have anything attach to the actual wheelchair – only the tray | The cup holder does not attach to the wheelchair. |
| I would like to have something easy to clean. | The cupholder is designed for ease of cleaning. |

In order to create a good design solution, it must be clear which needs are the most important to meet. While a list of needs was generated in response to the client’s words and actions, they likely cannot all hold the same weight. In fact, in some cases, given needs may contradict one another. Pursuant to this observation, it is important to quantifiably rank the importance of the given client needs.

To this end, all of the above needs will be ranked in terms of their relative importance on a scale of one to five, with one corresponding to the least important needs, and five corresponding to those needs which are highly important to satisfy. Table 10, shown below shows the ranking of each need statement’s importance, with a corresponding justification for each need statement’s placement.

Note that certain needs that were relatively redundant, namely “The cup holder is functional in design” and “The cup holder prevents the cup within from being knocked over” were combined into one need, seeing as they effectively describe the same need.

Table 12: Ranking of Need Statement Importance with Justification

| Need Statement | Importance (1-5) | Justification |
| --- | --- | --- |
| The cup holder prevents the cup within from being knocked over. | 5 | This need statement is fundamental to the functionality of the cup. The client stated that this was the principle role of the cup holder to be designed, and reiterated it several times throughout the meeting. Thus, failure to fulfill this need likely renders a given solution undesirable. |
| The cup holder can be used for long, uninterrupted periods of time. | 4 | It was stated by Travis (who supervised/ran the client meet) that Nicky uses her cup at all times. Thus, the ability of the cup holder to fulfill its function at all times is a very important consideration. |
| The cup holder can be used in a variety of locations. | 4 | Like the previous need statement, the use of the cup holder in different locales was also highlighted by the client(s). Consequently, it would appear that this need is also fundamental to a successful design solution. |
| The cup holder works well for the particular model of cup used. | 5 | It was highlighted during the meeting that Nicky exclusively uses one specific thermally insulated cup. Pursuant to this, the compatibility of the design solution with the given cup model is a pertinent consideration. |
| The cup holder is fixed in one position. | 3 | Given the body language of Travis and what he said, it appeared that he was not opposed to the cup holder having some range of movement. This being said, it appeared that he wished for the cup holder to be fixed to simplify the final prototype and design process. As a result, fulfillment of this need is not a strict requirement, but is likely advisable in order to please the client and streamline the design process. |
| The cup holder is positioned towards the back left corner of the wheelchair tray. | 4 | During the meeting several previous design solution prototypes were shown off. It was explicitly stated that one of the shortcomings of the previous iterations was incorrect cup holder placement on the wheelchair tray. Thus, a successful design solution should sit in the correct (specified) location on the wheelchair tray. |
| The cup holder is red | 1 | This need was iterated once, in passing by the client. Based on the phrasing of Travis, and his body language, he didn’t appear to think that the colour of the design solution was particularly consequential. |
| The cup holder has a straw holder. | 1 | Once again, this need was mentioned once in passing by the client. It was prefaced as being something that a perfect design that is all encompassing might have. |
| The cup holder is manufactured by 3D printing. | 5 | This was mentioned by the client several times, indicating a high degree of desire for the final design to fulfill this need. |
| The cup holder is attached to the wheelchair tray via a clamp. | 3 | Like the cup holder being fixed in one position, it appeared that Travis wished for this to be fulfilled because he thought that it would render the design simpler, and because in prior iterations it had worked. As such, it is a good need to fulfill, but it may be possible to use another approach provided the client is satisfied with it. |
| The cup holder does not attach to the wheelchair. | 4 | It was made rather clear that the preference is to only have the cup holder fixed to the wheelchair tray, and not to the wheelchair itself. As such, fulfilling this need is rather important. |
| The cupholder is designed for ease of cleaning. | 3 | Travis had mentioned that the cupholder did get dirty on occasion, and that it would benefit them to have a cupholder made from material that was easy to clean. |

# 6.0 Problem Statement

In order to design an adequate solution which pleases the client and fulfills their needs, a succinct, but comprehensive problem statement must be generated to guide the ideation process. To this end, the following preliminary statement was generated. Note that with iteration, and as the design process progresses it may become clear that the problem statement requires refinement. In such a case, the current problem statement may be altered to reflect a new understanding of the problem to be solved.

“Nicky requires a durable, functional cup holder to keep her cup in place and prevent her from knocking it over. The cup holder should be usable for long periods of time, in various places, and should be positioned towards the back corner of the wheelchair tray.”

# 7.0 Metrics & Benchmarking

By nature of the product to be designed, there are relatively few metrics which could be used to measure the established needs of the client. Nonetheless, the metrics below could be useful in measuring the adherence of a given design solution to the client’s needs. Table 11, included below shows the metrics (if any) associated with each need statement.

Table 13: Metrics Associated with Client Needs & Need Statements

| Need Statement | Metric | Unit of Measurement |
| --- | --- | --- |
| The cup holder prevents the cup within from being knocked over. | Applied lateral force to cup for which stability is maintained | Newtons [N] |
| The cup holder can be used for long, uninterrupted periods of time. | Length of time for which cup holder is held without falling off tray | Time [Hours] |
| The cup holder can be used in a variety of locations. | Cup holder is usable in a variety of locations | Binary [Yes/No] |
| The cup holder works well for the particular model of cup used. | Cup is held by cup holder | Binary [Yes/No] |
| The cup holder is fixed in one position. | Cup holder is fixed in one position | Binary [Yes/No] |
| The cup holder is positioned towards the back left corner of the wheelchair tray. | Cup holder position | Lateral distance from tray side & offset from tray back [Inches] |
| The cup holder is red. | Cup holder colour | Binary [Is red/Isn’t red] |
| The cup holder has a straw holder. | Presence of straw holder | Binary [Yes/No] |
| The cup holder is manufactured by 3D printing. | Percentage of components manufactured by 3D printing | Percentage [%] |
| The cup holder is attached to the wheelchair tray via a clamp. | Cup holder is attached via clamp | Binary [Yes/No] |
| The cup holder does not attach to the wheelchair. | Cup holder is attached to wheelchair | Binary [Yes/No] |
| The cupholder is designed for ease of cleaning. | Time to clean | Seconds [s] |

In terms of benchmarking, the following products will be benchmarked. They will be compared against the prescribed client needs and metrics to determine if they have the qualities of a valid solution to the design problem posed. What’s more they will offer insights into determining target specifications (both ideal and marginally acceptable). The products are summarized in Table 15, below. Two of the products are previous iterations of the cup holder to be designed, while the third is a less specialized, more general, commercially available equivalent product.

Table 14: Benchmarked Products (Description & Source)

| Product | Source (Hyperlink) |
| --- | --- |
| Kalolary Drinking Cup Holder This cup holder clips onto a table or surface using a powerful clip. It protrudes from the side of the table with a holder capable of holding drinks up to ~ 25 fl.oz.. It is applicable with many cup configurations, though appears to be especially geared towards mugs. Figure 3: Kalolary Cup Holder | Link |
| Steady Sip: This cup holder makes use of a 3D printed cup holder element attached to an extension arm, and a metallic clamp element. The product is a previous iteration of the cup holder to be designed. Figure 4: Steady Sip Cup Holder | Link |
| Cup Cruiser Crew: Similar in concept to the Steady Sip product above, this product makes use of many of the same principles. What is noteworthy is the fact that this design protrudes off of the tray, as opposed to protruding onto or over the tray. It also makes use of a 3D printed cup holder element fixed by an extension arm to a clamp element. Figure 5: Cup Cruiser Crew Cup Holder | Link |

To evaluate these designs, we use a decision matrix. We evaluate all of the above products using the specified metrics from above. We give a number from one through five (with five being highest) associated with a given product’s adherence to each of the metrics. We then give each metric a weighting, based on the relative importance of the need they represent. This weighting is multiplied by a given product’s score for that metric. Having calculated each product’s weighted score for each metric, the scores are added up to determine the product which best aligns with the client’s stipulations.

Note that certain metrics simply cannot be measured without actually possessing the product. In certain cases (such as for applied lateral force) it may be possible to make an educated guess about each products performance based on a variety of factors, such as material selection, cup holder geometry and depth, etc. In other cases, assessing each product’s performance is altogether impossible. For instance, without physically possessing each product, assessing the length of time for which they can hold a cup without falling is not possible. In such cases, each product will be given an equal score of three such that the results of the benchmarking are not swayed by crude guesswork.

Table 15: Decision Matrix for Benchmarked Products

| Criteria/Metric | Weighting | Kalolary |  | Steady Sip |  | Cup Cruiser |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Applied lateral force to cup with stability maintained | 0.175 | 3 | 0.525 | 4 | 0.7 | 4 | 0.7 |
| Length of time for which cup holder is used without falling | 0.08 | 3 | 0.24 | 3 | 0.24 | 3 | 0.24 |
| Cupholder can be used in a variety of places | 0.08 | 3 | 0.24 | 3 | 0.24 | 3 | 0.24 |
| Works well for particular cup model | 0.1 | 1 | 0.1 | 5 | 0.5 | 4 | 0.4 |
| Cup holder is fixed | 0.05 | 5 | 0.25 | 5 | 0.25 | 5 | 0.25 |
| Cup holder is positioned correctly | 0.08 | 2 | 0.16 | 3 | 0.24 | 1 | 0.08 |
| Cup holder is red | 0.015 | 1 | 0.015 | 5 | 0.075 | 1 | 0.015 |
| Cup holder has straw holder | 0.015 | 1 | 0.015 | 1 | 0.015 | 1 | 0.015 |
| Cup holder manufactured by 3D printing | 0.175 | 1 | 0.175 | 3 | 0.525 | 3 | 0.525 |
| Cup holder is attached to tray via clamp | 0.05 | 1 | 0.05 | 5 | 0.25 | 5 | 0.25 |
| Cup holder does not attach to wheelchair | 0.1 | 5 | 0.5 | 5 | 0.5 | 5 | 0.5 |
| Time to clean | 0.08 | 3 | 0.24 | 4 | 0.32 | 4 | 0.32 |
| Total | 1 | 2.495 |  | 3.855 |  | 3.535 |  |

As one might imagine, the purpose-designed cup holders, which were previous iterations of the product to be designed, performed better than the mass-produced cup holder. Out of the three benchmarked products, the Steady Sip cup holder performed best, setting the example of what elements a successful design might encompass.

Notably, however, there were metrics for which every single product struggled. For instance, none of the cup holders were quite positioned correctly. The Steady Sip, for its part, protruded to far onto the tray, as noted by the client. Conversely, the other two cup holders hung off the edge of the tray and would not sit atop it, as specified by the client. What’s more, none of the products contained a straw holder as the client indicated might be helpful.

As such, though some of the products represented good starting points for a successful concept, additional work and refinement is necessary to create a truly successful design.

# 8.0 Development of Target Specifications

Having developed metrics and benchmarked similar products, the final step in creating a framework for evaluating subsequent design solutions is the creation of target specifications. Using information gathered in the benchmarking phase, and using the design team’s judgement (with relation to what was said by the client) a set of both ideal, and marginally acceptable values for each metric will be developed.

The first metric which must have a target specification associated with it, is the force to be resisted by the cup holder. The force necessary to knock the cup over (without any additional support or holder) can be calculated using some basic statics. The Free Body Diagram of the situation is drawn, as depicted in Figure 6 (below).

Figure 6: Free Body Diagram of Cup

As one can see, the two applied forces are the applied force (that which knocks the cup over) and the gravitational force (weight) of the cup. The moments from the two forces must cancel each other out in order for the cup to remain stable. The cup (based on some very rudimentary research and preliminary google searches) is assumed to have a base radius of 1.25” or 31.75 mm. It is assumed to have a mass of 0.3 kg, and the applied force is assumed to be acting 100 mm above the base of the cup. The following equation can be written to describe the moment balance in the given scenario. Subsequently, FApplied may be solved to determine the force required to overcome the equilibrium condition and knock the cup over. Moments are taken about the lower right corner of the cup’s profile.

FApplied100 mm=mCupg31.75 mm

FApplied=0.3 kg9.81 m/s231.75 mm100 mm=0.9344025 N

Hence, a force of roughly a single Newton would be sufficient to knock over the unsupported cup. To ensure excellent stability of the cup, a factor of safety should be introduced in determining the applied force which the cup holder should be able to resist.

Based on group deliberation, a factor of safety of 25 was determined to be marginally acceptable, while a factor of safety of 35 was determined to be ideal. Hence:

FResisted Marginally Acceptable=25 N

FResisted Ideal=35 N

With respect to the time that the cup holder must hold itself on the tray without falling, upon deliberation, 24 hours was selected as a reasonable and informed ideal target spec. It isn’t completely feasible for the cup holder to be expected to hold itself on the tray indefinitely, but it should be able to hold itself on the tray for a very long period of time. Thus 24 hours seems reasonable. As for a marginally acceptable value, deliberation dictated that 12 hours was a good choice, seeing as it still represents a considerable amount of time, but less than 24 hours. In an ideal world, the cup holder should be able to be fixed to the wheelchair tray and held in place all day before potential removal at night, which would be possible in the case of the ideal value. This however, would not be possible in the case of the marginally acceptable value, in which case the cup holder may need to be re-attached or adjusted at some point during the day.

In terms of the target specification for the percentage of the product which is 3D printed, it became evident in conversation with the client that ideally, the entire product (100%) would be 3D printed. In saying this, it was made clear that they were willing to make concessions regarding the material used for the clamp and other elements of the product, explaining that the fact that the cup holder element itself was 3D printed was of the greatest importance. As such by parts number, having 20% of the product manufactured by 3D printing would likely fall within marginally acceptable limits, considering the client spoke favourably about past designs which incorporated metal elements and fasteners in connecting the cup holder to the fixation mechanism.

In the case of metrics that are binary in nature, the general approach to selecting a target specification is to say that satisfaction of the metric is ideal, and failure to satisfy the metric is marginally acceptable. This approach works in the case of metrics associated with needs of lesser importance, such as cup holder colour, or the presence of a straw holder. However, in the case of more important needs and associated metrics, the ideal and marginally acceptable values for a given target specification may be one and the same. This is to say that certain metrics and associated needs must be fulfilled for a given design solution to be considered successful. Table 17, included below indicates the ideal and marginally acceptable values for target specifications related to binary metrics. Note that the metrics for which the ideal and marginally acceptable values are the same are associated with needs and need statements of high importance.

Table 16: Target Specifications for Binary Metrics

| Metric | Ideal Value | Marginally Acceptable Value (May be same as ideal value) |
| --- | --- | --- |
| Cup holder is usable in a variety of locations | Yes | Yes |
| Cup is held by cup holder | Yes | Yes |
| Cup holder is fixed in one position | Yes | No |
| Cup holder colour | Is red | Isn’t red |
| Presence of straw holder | Has straw holder | Doesn’t have straw holder |
| Cup holder is attached via clamp | Yes | No |
| Cup holder is attached to wheelchair | No | No |

Having individually justified and selected target specifications for each metric, the below table (Table 18) summarizes the selected target specifications, both insofar as their ideal values and their marginally acceptable values.

Table 17: Target Specifications Associated with Metrics

| Metric | Unit | Ideal Value (with relation) | Marginally Acceptable Value (with relation) |
| --- | --- | --- | --- |
| Applied lateral force to cup with stability maintained | [N] | > 35 N | > 25 N |
| Cup holder is usable in a variety of locations | Binary | = Yes | = Yes |
| Cup is held by cup holder | Binary | = Yes | = Yes |
| Cup holder is fixed in one position | Binary | = Yes | = No |
| Length of time for which cup holder is used without falling | [Hours] | > 24 Hours | > 12 Hours |
| Cup holder colour | Binary | = Red | ≠ Red |
| Presence of straw holder | Binary | = Present | ≠ Present |
| Cup holder is attached via clamp | Binary | = Yes | = No |
| Cup holder is attached to wheelchair | Binary | = No | = No |

# 9.0 Concept Development Process

With respect to the concept development process, structured, but relatively unrestricted brainstorming and sketching will be used as the methodology for coming up with ideas. Firstly, the entire product will be broken down into subsystems, for which concepts can be generated. In synthesizing and combining these subsystems, a series of global design solutions can be generated. In selecting the best few global solutions, and evaluating them based on the generated target specifications, a final, ideal, global solution can be selected.

## 9.1 Definition of Product Subsystems

The product to be developed can largely be divided into the following three subsystems:

- Tray connection element

- Connection between tray connector and cup holder body

- Cup holder body

While more subsystems could be defined, it would likely serve little purpose to do so. Ultimately, the generation of subsystems serves to simplify the concept generation process as much as possible. Seeing as the product to be developed, a cup holder, is relatively simple (at least in relation to many other commercially available products) creating an excessive number of subsystems would not streamline the process, and in fact, could make it more difficult to create a coherent design combining all of the specified subsystems.

After group deliberation the following table (Table 19) was generated to summarize the potential design solutions to fulfill each subsystem. Note that additional ideas were generated, but all ideas were vetted and scrutinized after brainstorming was complete. The result is the shortened list of concepts presented.

Table 18: Potential Design Solutions to Proposed Product Subsystems

| Subsystem | Solutions | Additional Notes |
| --- | --- | --- |
| Tray Connection Element | Clip (similar to Kalolary cup holder) | Clip uses springs to clamp down on end of wheelchair tray. |
|  | Clamp (as seen in previous design solution iterations) | Clamp makes use of some sort of adjustable thumb screw to clamp down on the wheelchair tray from underneath. |
|  | Suction cup | A suction cup is attached to the underside of the cup holder mechanism and suctions onto the wheelchair tray. |
| Cup Holder & Tray Connection Joint | Rigid connection | A rigid bar of material is connected between the fixation portion of the mechanism and the cup holder portion. |
|  | Segmented connection | A segmented, highly adjustable connecting rod is connected between the two other subsystems. This mechanism is often seen in desk lamps, as pictured in the below figure (Figure 7). Figure 7: Desk Lamp with Segmented Arm |
|  | Hinged Connection | Similar to the above idea (in that it is also inspired by desk lamps) A hinged connecting arm is used to connect the fixation portion of the mechanism to the cup holder portion. Similar to the mechanism in Figure 8. Figure 8: Desk Lamp with Hinged Arm |
| Cup Holder Body | 3D printed cup holder (removable from connection mechanism) | Using CAD (Computer Aided Design) tools, design a 3D printable cup holder element that can be detached from the rest of the product. Similar in nature to previous years design solutions. |
|  | Purchased commercial cup holder element | Purchase a commercially available cup holder element and fix it to the rest of the product using fasteners of some kind. |
|  | Metal wire cup enclosure | Create a minimalist cup holder element using bent metal wire elements wound in the profile of the cup. |

## 9.2 Global Concept Generation

Having generated, and whittled down, solutions for each subsystem it is now possible to combine said subsystem solutions into a series of global product design solutions. The method selected for this process was to sketch out sample concept ideas and evaluate them using the aforementioned and defined target specifications. The global design solutions generated are enumerated upon in the below figures and passages. As with the subsystem design process, many designs were generated with a preliminary run through being used to clear out designs with obvious flaws and shortcomings, so as not to waste time and resources evaluating concepts that plainly would not meet the client’s requirements. What’s more any design solutions that were essentially the same were coalesced into a single concept, reducing redundancies between proposed design solutions.

The first design solution is pictured below, in Figure 9.

Figure 9: First Global Concept Sketch

The concept includes a clip for fixation of the product to the wheelchair tray. It makes use of a rigid connecting “rod” between the clip and the cup holder body. The cup holder body is to be made from 3D printed material, is to be removable from the connecting rod, and is molded to the shape of the client’s cup. The clip mechanism is to be sourced from a third party, and the connecting rod is to be 3D printed.

The next concept to be explored is presented in Figure 10 below.

Figure 10: Second Global Concept

The concept makes use of a clamp (not pictured) to affix the product to the wheelchair tray. It also makes use of a rigid connecting rod to connect the clamp portion of the product to the cup holder portion. The cup holder portion of the product is to be 3D printed, with an angled straw holder incorporated into the design, facing towards the center of the tray for easy access. This particular design incorporates a drainage surface on the bottom of the cup holder compartment to funnel any spillage or condensation from cold beverages. When the liquid is funneled through the drainage, it is collected into a removable drip pan for quick disposal of its contents. The drip pan features a simple locking mechanism to keep it in place. The remainder of the product could be constructed from either 3D printed PLA (or an equivalent material) or metal. Only with additional prototyping and testing will it be possible to determine which material combination is optimal.

The third design, presented in Figure 11, is similar in nature to the previous design, in that it incorporates many of the elements of the previous design.

Figure 11: Third Global Concept Sketch

As shown in the above sketch, the concept is essentially the same as that above, but with the method of fixation changed from a clamp to an adhesive, or a suction cup of some kind.

The final potential design is shown in Figure 12, below. It makes use of a hinged connector to connect the 3D printed cup holder element and the clamp fixation portion of the cup holder.

Figure 12: Fourth Global Concept Sketch

Having created the former four designs, they must now be evaluated using a decision matrix. The decision matrix in question is shown in the table below (Table 20). Note that in certain cases, it is not possible to precisely determine the relative performance of the different concepts. For instance, it is not possible to determine the force which each cup holder to resist, but professional judgement can be employed in order to determine that based on fixation method for example, certain concepts would likely perform better. Pursuant to this, it may be logical to conclude that a clamped concept would resist a greater force than a clipped one.

Table 19: Decision Matrix for Global Concept Evaluation & Selection

| Criteria/Metric | Weighting | Concept#1 |  | Concept#2 |  | Concept#3 |  | Concept#4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Applied lateral force to cup with stability maintained | 0.175 | 3 | 0.525 | 5 | 0.875 | 2 | 0.35 | 5 | 0.875 |
| Length of time for which cup holder is used without falling | 0.08 | 2 | 0.16 | 4 | 0.32 | 5 | 0.4 | 4 | 0.32 |
| Cupholder can be used in a variety of places | 0.08 | 3 | 0.24 | 3 | 0.24 | 4 | 0.32 | 3 | 0.24 |
| Works well for particular cup model | 0.1 | 4 | 0.4 | 4 | 0.4 | 4 | 0.4 | 4 | 0.4 |
| Cup holder is fixed | 0.05 | 5 | 0.25 | 5 | 0.25 | 5 | 0.25 | 5 | 0.25 |
| Cup holder is positioned correctly | 0.08 | 4 | 0.32 | 4 | 0.32 | 5 | 0.4 | 4 | 0.32 |
| Cup holder is red | 0.015 | 5 | 0.075 | 5 | 0.075 | 5 | 0.075 | 5 | 0.075 |
| Cup holder has straw holder | 0.015 | 1 | 0.015 | 5 | 0.075 | 1 | 0.015 | 1 | 0.015 |
| Cup holder manufactured by 3D printing | 0.175 | 5 | 0.875 | 5 | 0.875 | 4 | 0.7 | 5 | 0.875 |
| Cup holder is attached to tray via clamp | 0.05 | 1 | 0.05 | 5 | 0.25 | 1 | 0.05 | 5 | 0.25 |
| Cup holder does not attach to wheelchair | 0.1 | 5 | 0.5 | 5 | 0.5 | 5 | 0.5 | 5 | 0.5 |
| Time to clean | 0.08 | 4 | 0.32 | 5 | 0.4 | 4 | 0.4 | 4 | 0.4 |
| Total | 1 | 3.73 |  | 4.58 |  | 3.86 |  | 4.52 |  |

Based on the decision matrix implemented above, it is clear that the second global concept, is most in line with the client’s needs. As such, it will be selected as the basis for any and all concepts and prototypes that are to follow.

## 9.3 Justification of Global Concept Solution, and Concept Revision

It makes sense that the decision matrix showed concept#2 to be the optimal concept. Logically, the clamp would allow good resistance to knocking, and allow the cup holder to be fixed to the tray for long periods of time. It employs a straw holder, is red, and has a flared shape which is characteristic of the client’s cup. As such, it seems well suited to tackling the problem posed.

In order to garner additional feedback from the client during the meet, the selected global concept will be used to create three variants for presentation to the client. The figures below detail the different variations created. Note that the primary purpose of these variations is to generate conversation, and either confirm a solid understanding of the problem, or rectify any misunderstandings with respect to the design problem.

The first variant, presented in Figure 13 below, features a distinct open horizontal straw holder allowing for quick access and easy cleaning. It also features a detachable napkin holder, making use of the outwards extension of the straw holder, and a removable drip pan for drainage of leaking liquids.

Figure 13: Selected Global Concept - First Variant

The second variant, presented in Figure 14 below, features an angled straw holder with three different sized holes for different-sized straws. Like the first variation there is a drip pan located below the cup-holding compartment.

Figure 14: Selected Global Concept - Second Variant

The third variant, presented in Figure 15 (next page), has a distinct rotatable straw holder positioned at an angle for a compact design. Like the other variants, it includes a short bar to attach to the clamping compartment.

Figure 15: Selected Global Concept - Third Variant

## 9.4 Relation of Global Concept Selection to Target Specifications & DFX Modalities

As can be seen by the modular design of the proposed concept variations, the proposed concept is in line with the DFX outlined above. The creation of the product in several pieces will render assembly and manufacturing easier, seeing as components can be manufactured individually using well understood manufacturing methods (3D printing, metal forming, etc.) and assembled using standard fasteners. This also aids with repairability, seeing as individual components can be repaired without having to repair or dispose of the product as a whole.

In breaking the product up into smaller components, it may also be possible to reduce material costs, and employ manufacturing methods that are cost saving in nature. In this manner, design to cost principles are adhered to.

With respect to target specifications, specific adherence to the outlined values is not possible at this stage, seeing as many of the specifications will require testing. This being said, as mentioned during the concept selection process, logical thinking and evaluation suggests that the various concepts are in line with the target specifications.

Ultimately, a rigorous testing process will be necessary in order to ensure compliance of generated concepts (and subsequent prototypes) with target specifications.

# 10.0 Detailed Design and Design Refinement

Having selected a design (or several variations of a given design concept), it is now possible to refine the design and create a detailed design from which to base future prototypes, and the final design solution. To do this, the first step is reflecting upon client feedback gathered during the second client meeting.

## 10.1 Client Feedback Summary

It must be said that the client offered relatively little in the way of meaningful feedback during the meeting. Though the client was presented with three potential variations on a design solution, they were relatively indecisive in their ability to select one as being their favourite. They reiterated that the principal thing they were looking for in a given design is functionality. To this end, it was possible to prompt the client using questions, and determine that there preferred approach to solving their design problem posed, was the fulfillment of the design pictured in Figure 15. Seeing as this design contained the least elements, and was overall the least sophisticated, it was agreed that this design should constitute the basis for the first prototype, with the other designs (which included more elements, such as liquid catch trays) being explored in future prototypes, if time permitted. This strategy aligned best with the client’s stress of the importance of a functional prototype.

Overall, the client had few overtly negative things to say about any of the provided designs. One cautionary observation of the client that bears noting is their reiteration of the straw holder element as being an “add-on”. They reiterated that it was a thought that they came up with on the spot, and that it constituted an aspect of the product that was not core to its functionality. Hence, it stands to reason that the straw holder should be removed from the design, with the potential to re-add it if time permits.

As such, the finalized design concept is essentially the design proposed in Figure 15, with the straw holder element removed. Upon team deliberation, it was determined that the cup holder would be secured by a C-clamp to secure the cup holder to the tray. This element would be manufactured using metal (likely stainless steel) in Brunsfield, and attached to the cup holder element using an additional metal bracing. All pieces were to be secured by standardized fasteners, to be purchased as necessary.

## 10.2 Detailed Design Concept

The following is the specific, detailed design upon which all subsequent prototypes, and the finished product, are to be based. Note that upon further research of past design solutions, it was decided to purchase the clamp used to fix the cup holder to the wheelchair tray. The fasteners, for their part, constitute standard components and thus are not examined in great detail in the subsequent schematics. Further details, including links to sources, are available for all of the purchased parts. Such information is located in section 11.7, in the detailed product BOM.

As was detailed earlier, and as can be seen in the design drawings below, the product consists of three interconnected subsystems; the clamp, the cup holder, and the connecting brace. The cup holder is additively manufactured using 3D printing. It sits atop the stainless-steel housing of the C-clamp element. The two components are connected via standard fasteners as shown. A vertically oriented metal brace is affixed to both the cup holder element and the C-clamp housing, adding rigidity to the structure. This bracing is equally held on using standard fasteners.

As a result of the aforementioned structure of the product, it should theoretically be able to fulfill all of the client’s demands.

It is important to note that this detailed design is subject to change. Though it was carefully thought out prior to being implemented, changes in client desires, or results of design reviews could dictate that a better design becomes readily apparent. In such cases, it would be foolish to forego a superior design for the sake of continuity of a flawed one.

Figure 16: Cup Holder Detailed Drawing

Figure 17: C-Clamp Base Detailed Drawing

Figure 18: Clamp Sub-Assembly

Figure 19: Assembly Drawing of Final Proposed Design

## 10.3 Design Considerations Regarding Fulfillment of DFX Factors

There are a number of considerations to be made in the fabrication of prototypes, and the finished product, particularly in respect to the fulfillment of the prescribed DFX factors. One of the foremost of these considerations, is the selection and use of standard fasteners. Such fasteners allow the creation of a modular design, which aligns with design for manufacturing, and design for assembly principles. What’s more, these fasteners are typically cost effective and readily available, therefore agreeing with principles of both design to cost and design for repairability. As a result, one can see that the use of standard fastener elements has a positive bearing on the fulfillment of the majority of selected DFX factors.

Similarly, the use of 3D printing technology will be core to the fulfillment of selected DFXs. Seeing as the client has access to 3D printers, the use of such technology greatly increases their ability to repair the product in the event of damage, thus aligning with design for repairability principles. What’s more, as discussed in the sustainability report above, 3D printing technology is exceptionally cost effective for small scale productions. At the current point in the product’s lifetime this could greatly increase the chances of adhering to the prescribed $50 budget. Thus, 3D printing aligns with design to cost principles too. It should furthermore come as no surprise that 3D printing also lends itself to design for manufacturing seeing as it allows for (relatively) rapid manufacturing of rather complex geometries. Thus, though the use of 3D printing was already a near forgone conclusion (based on the client’s stipulations) it nonetheless constitutes an important consideration in the adherence of the product and its development process to key DFX principles.

As discussed earlier, some of the preliminary designs included “extra” additions. While, the re-addition of these items may be appealing, particularly in the event of a time surplus, it is important to be measured in the implementation of said elements. Ultimately, while beneficial at a glance, adding too many of these extra features could (but may not necessarily) have the effect of cluttering the final design. Additional components could be difficult to manufacture (and will by default increase overall product manufacturing time). What’s more, having a great deal of components making up the final product could have the effect of increasing assembly time or rendering assembly increasingly difficult. As a result, the re-addition of intentionally omitted design elements could have an adverse effect in respect to the implementation of design for manufacturing and design for assembly principles.

In so far as cost, additional product components would likely result in increased product manufacturing and development costs, and could also hamper the ability of the client to swiftly and easily repair broken components, thus having an adverse effect on design to cost and design for repairability principles.

Finally, it is important that any product “add-ons” do not hamper the overall functionality or ease of use of the product. For instance, the re-addition of a straw holder should not affect the weight distribution of the cup holder, and make the structure and cup held within easy to knock over. As such, considerations must be made with respect to the effect of product “add-ons” on product usability.

## 10.4 Relevant Skills and Resources to be Used

In order to complete the product, it will be necessary to leverage a number of engineering resources and skills.

The foremost resource to be leveraged will be the Makerspace and the resources within. Most important among these resources, are the 3D printers. These machines will be necessary for the manufacturing of the cup holder element as specified by the client, and the detailed design proposed above. In order to create the geometry to be printed, it will be necessary to leverage acquired CAD skills. Using SOLIDWORKS, or an equivalent software, the cup holder’s geometry must be specified, before the Cad file can be downloaded, and ultimately, printed. No other skills will likely be necessary for the completion of the cup holder element.

Fasteners will be purchased, and thus will also require no acquired skills to implement. These elements, however, will require consultation of the correct retailers (resources) in order to ensure the quality of purchased goods. For fasteners, consultation of a dedicated mechanical component retailer (such as McMaster-Carr, or Home Depot) is likely advisable.

In order to manufacture the bracing piece, and the body of the C clamp, it will be necessary to make use of the Brunsfield Centre’s facilities. Basic metal machining will be necessary to create the required geometry and to tap the threaded holes for any fasteners. Understanding of most of the machinery contained within the Brunsfield Centre (including most of that which will likely be required to create the clamp) is lacking. As such, a concerted effort will have to be made in order to acquire knowledge regarding the tools available at the Brunsfield Centre and their application. Such skills can be attained through attending one of CEED’s many workshops held throughout the semester. As such, updates regarding the scheduling of CEED workshops should be closely monitored.

## 10.5 Project Timeline

In order to successfully complete the project by design day (the prescribed “due date”) adequate time management and planning is necessary. The following represents an estimate of the time required to implement and/or create each aspect of the product.

Table 20: Implementation Plan

| Week (Actual Available Time) | Tasks to be Completed (Including Time Estimates for Individual Tasks and Subtasks) |
| --- | --- |
| Feb 2 - Feb 8 | Deliverable D: - Finalize design concepts to ensure all feedback and requirements from second client meet are incorporated (2 days) - Source materials and create a bill of materials (5 days) - Review deliverable (< 1 day) - Update project plan (< 1 day) - Submit Deliverable (Milestone) - Attend Design Review (Milestone) Deliverable E: - Divide up deliverable tasks (< 1 day) |
| Feb 9 - Feb 15 | Prototype Manufacturing and Assembly - 3D Print Cup Holder Subassembly Prototype (1 day – Ideally no later than Monday so testing can commence as soon as possible) - Test fit of cup in cup holder element using Kaitlynn’s cup which is identical to the client’s model (< 1 day) - Test stability of cup within cup holder (while fixed) in response to various forces (2 days) Deliverable E (occurs in parallel with prototype creation): - Work on deliverable (time varies according to individual – each person responsible for managing their own time) - Review deliverable (< 1 day) - Update project plan (< 1 day) - Submit deliverable (Milestone) Progress Presentation: - Summarize Previous Deliverables (2 days happens in parallel with prototype creation) - Repurpose client meeting & design review slides for efficiency - Summarize results of prototype testing (1 day) - Practice presentation parts (1 day) - Perform Progress Presentation (Milestone) Peer Feedback: - Complete “Peer Feedback and Team Dynamics (<1 day) managed by individuals |
| Feb 16 - Feb 22 (Reading Week) | Second Prototype Manufacturing and Assembly: - Discuss positives & negatives of first prototype (< 1 day) - Double-check BOM (< 1 day) - If validated, purchase fasteners, and materials (< 1 day) - Begin Machining Clamp Elements (5 days) - Note that training will likely be necessary, in which case this step may need to be pushed back Deliverable F (occurs in parallel with prototype development): - Divide up tasks for deliverable (< 1 day) - Prepare questions and presentation slides for client meet (< 1 day) - Begin work on deliverable (time varies according to individual – each person responsible for managing their own time) - Identity non-functional design constraints (< 1 day) - Summarize and document most recent prototype (<2 days) |
| Feb 23 – March 1 | Deliverable F: - Summarize work completed over the reading week (<1 day) - Complete additional testing (if needed) (<2 days) - Review deliverable (1 day) - Update project plan (< 1 day) - Submit deliverable (Milestone) Second Prototype Assembly: - Finalize any CAD drawings (1 day) - Gather and 3D print all necessary parts for prototype 2 assembly (< 2 days) Deliverable G - Divide up work for deliverable (< 1 day) |
| March 2 – March 8 | Second Prototype Assembly & Testing: - Finish Assembling Second Prototype (Clamp & Bracing elements) (2 days) - It should be permissible to not quite have assembly completed, but manufacturing must be completed - Test Clamp fixation time (1 day) Client Testing and Feedback: - Join prototypes 1 & 2 ahead of client meeting (1 day – ideally could be done the week prior) - Test holistic product (2 days – prior to meeting) - Attend Client Meeting (Milestone) Deliverable G: - Complete deliverable (time varies according to individual – each person responsible for managing their own time) - Create list of costs (<1 day) - Determine Break even point (<2 day) - Developing a 3-year income statement (<2 days) take time to understand what NPV analysis is - Create list of economical assumptions (<1 day) - Research and identify 2 intellectual properties (<1 day) - Update Project Plan |
| March 9 – March 15 | Refine Prototype Based on Client Feedback - In the event of negative feedback make alterations to prototype (LAST RESORT - timeline will be determined after we receive client feedback) - Note that at this point there should not be significant negative feedback, as the prototype should closely resemble the desired product - Make minor alterations to prototype in the event of measured feedback/criticism (1-2 days – timeline could vary based on feedback) - Devise tests for any alterations to prototypes as needed (< 1 day) Deliverable G: - Complete deliverable (time varies according to individual – each person responsible for managing their own time) - Create list of costs (<1 day) - Determine break-even point (<2 day) - Developing a 3-year income statement (<2 days) take time to understand what NPV analysis is - Create list of economic assumptions (<1 day) - Research and identify 2 intellectual properties (<1 day) - Update Project Plan Review deliverable (< 1 day) - Update Project Plan (< 1 day) - Submit deliverable (Milestone) Deliverable H: - Divide up tasks (< 1 day) - Discuss Pitch Layout (< 1 day) Deliverable I: - Divide up tasks (< 1 day) - Note that this deliverable is exceptionally long and thus must be started early, hence the early division of tasks - Discuss video layout (< 1 day) - Review User Manual Layout (< 1 day) |
| March 16 – March 22 | Final Prototype Testing and Implementation: - Implement any testing protocols for last minute alterations - Make any finishing touches to product (Variable timeline) - As before, changes made at this stage should not be significant; product should largely be finished - Declare product finished (Milestone) Deliverable H: - Prepare pitch (3 days) - Create a summary slide - Divide up who will explain user requirements, explain importance of problem, benchmarking, and why our design is the best - Assemble Design Day presentation materials (5 days – can be done largely in parallel with pitch) Deliverable I: - Ask for team update (< 1 day) - Film Video (<2 days) |
| March 23 – 26 | Deliverable H: - Finalize pitch & presentation materials (1 Days) - Review deliverable (< 1 day) - Update Project Plan (< 1 day) - Submit deliverable (< 1 day) Discuss Design Day Elements - Table set-up (< 1 day) - Attire (< 1 day) - Practice design-day pitch (practice over span of 2 days) |
| March 27 | DESIGN DAY PRESENTATION (MAJOR MILESTONE) |
| March 28 – March 30 | Deliverable I: - Continue work on user manual using template provided (2 days) - Edit Video (1 day) - Review deliverable (< 1 day) - Submit deliverable (< 1 day) |

## 10.6 Critical Assumptions

Several critical assumptions must be made during the design process of the cup holder, which may affect the success of the project’s implementation. The first is that the design assumes that the cup holder will fit the client’s specific cup based on measurements taken during the first client meeting. This can significantly impact the functionality of the design if those measurements are not entirely accurate, or if the cup holder does not accommodate for any variations in the client’s cup.

It is also assumed that sufficient PLA (the material selected for 3D printing) will be readily available and that it will have the necessary strength and durability to withstand regular use and repeated cleaning. If there is not enough PLA, or if the filament ends up being weaker than expected, the cup holder will not perform as needed and may be subject to wear and failure over time. Generally speaking, (based on previous experience with the Makerspace) availability of PLA is not a great issue.

The assumption that hardware components (i.e. nuts, bolts, screws, etc.) will be readily available for purchase may also impact the implementation of the design. Any difficulties obtaining these components or keeping their cost within the given budget may affect the completion of the prototype and require the team to find alternate solutions.

Another critical assumption is that the necessary materials and equipment will be available to machine a C-clamp in the Brunsfield workshop, and that this process will result in a reliable, high-quality component for use in the prototype. It is further assumed that this clamp will provide enough clearance to firmly attach to the tray itself, rather than the wheelchair, to ensure a secure fit that will not loosen over time or due to impact. If the clamp does not have a proper attachment to the wheelchair tray, the cup holder may not stay securely in place. Given that one of the client’s most important criteria is that the product resists being knocked over, any failure in this aspect will negatively impact the cup holder’s functionality and reliability.

It must also be noted that there is an inherent assumption that there will be ready access to the Makerspace’s facilities (namely the 3D printers). These devices are typically in high demand, particularly towards the end of a given semester as design day nears. As such, though in theory access to a 3D printer is always there, in practice, it is often difficult to actually obtain such access. In order to increase the likelihood of having rapid access to a 3D printer, the sizing of the detailed design (as outlined above) should be adhered to. The cup holder element in this case, should be small enough to fit in some of the smaller 3D printers in the Makerspace (of which there are many), thereby increasing the likelihood of a timely print. Arriving at the Makerspace early (as soon as it opens) is another viable strategy in ensuring rapid printing of components.

## 10.7 Product BOM

Illustrated in the subsequent table is the detailed BOM for the development of the product. All relevant information regarding materials to be purchased is included. This includes information regarding suppliers, part costs, quantities, and images. Note the inclusion of all necessary materials, even those that do not have an actual cost associated with them. Note that rates for stainless steel metal (to be used in the bracing and C clamp) are not specified on the Makerstore website. As such, consultation of CEED personnel will likely be necessary to determine an exact price. Though the amount of metal to be used is relatively small (on the order of a few hundred grams), a conservative estimate of $15.00 will be applied for the price of this element. It should be noted, however, that there is sufficient room in the budget to allow for material costs of up to nearly $25.00 (or half the project budget).

All other costs are absolute in nature; they represent the fixed cost of acquiring the specific product specified. In purchasing all of the below elements from retail locations, costs were able to be reduced significantly, seeing as shipping costs will not be applicable. This constitutes a minorly inconvenient, but creative way to reduce overall project cost.

Table 21: Detailed BOM for Product

| Category | Supplier | Part Name | Qty | Image | Unit Cost | Total Cost |
| --- | --- | --- | --- | --- | --- | --- |
| Fastener | McMaster Carr | High-Strength Class 10.9 Steel Hex Head Screw Link | 1 |  | $16.95 + HST (for a pack of 10) | $19.15 (including HST) |
| Fastener | Home Depot | Paulin M10-1.50 x 25mm Class 8.8 Metric Hex Cap Screw - DIN 933 Full Thread - Zinc Plated Link | 2 |  | $1.42 + HST | $3.21 (including HST) |
| Fastener | Home Depot | Paulin M8-1.25 x 20 mm Class 8.8 Zinc Plated Hex Bolt Link | 1 |  | $0.77 + HST | $0.87 (including HST) |
| Material | Makerspace/CEED | PLA filament | 300g |  | FREE | FREE |
| Fastener | Home Depot | Paulin M10-1.50 Class 8 Metric Hex Nut-DIN 934 - Zinc Plated Link | 3 |  | $0.39 + HST | $1.32 (including HST) |
| Fastener | Home Depot | Paulin M8-1.25 Class 8 Metric Hex Nut-DIN 934 - Zinc Plated Link | 1 |  | $0.34 + HST | $0.38 (including HST) |
| Material | Brunsfield Centre | Stainless Steel | 300g |  | Variable (Assume $15.00 based on low volume of metal required) | $15.00 |
| Fastener | Home Depot | Paulin M8 Metric Lock Washers - Zinc Plated Link | 1 |  | $0.18 + HST | $0.20 |
| Fastener | Home Depot | Paulin M10 Metric Lock Washers - Zinc Plated Link | 2 |  | $0.20 + HST | $0.45 |
| TOTAL COST: |  |  |  |  |  | $40.58 |

# 11.0 Project Plan Update

The following figure (subsequent page) depicts the project plan update, in the form of the updated project Gantt chart. Note that there have been minimal changes to the project schedule since the previous deliverable submission (which was only one week ago). As the design process progresses further, more significant alterations to the project plan may be expected.

Figure 20: Project Plan (Gantt Chart) Update

# 12.0 Conclusions

In performing the above analysis of sustainability principles, product life cycle, and design for X principles, it was possible to create a preliminary framework for the design of a successful product.

Using a three-pronged sustainability analysis of the economic impact, the social impact, and the environmental impact it was possible to understand the positive and negative impacts of the designed product. The subsequent Life Cycle Assessment was aimed at understanding the ecological impact of the product’s entire lifespan, from cradle to grave. Through allocating points based on the impact of acquiring the materials, processing the materials, manufacturing, packaging and transportation it was possible to find the impact of an equivalent product, and potentially reduce the impact of the product to be designed.

Based on the client meet DFXs were selected with the goal of understanding how to better serve the client in the product development process.

Taking all of the above into account, it was possible to take the proper time and effort to empathize with the client and understand their problem. From here, concepts were generated, with a final global concept (with several variations) being selected for presentation to the client.

In the end, it was possible to see the importance of taking the time to fully understand the problem to be solved. In being methodical, and resisting the urge to forge ahead to the design process, several high quality concepts were generated.

After pitching said high quality concepts, it was possible to gain valuable client insight, which guided the creation of a detailed design. Ultimately, the detailed design above was decided upon, after careful deliberation. Great care and planning was used in planning out implementation of the various aspects of the detailed design, including the creation of a detailed BOM and consideration of resources to be used in product development, as well as timeline for product creation.

Having created a detailed design, the next steps are to create the first prototype of the product and perform tests on it to ensure functionality of the final product. At this stage ironing out any kinks with the preliminary detailed design and forging a path towards a high quality finished product are key to project success.

# Bibliography

[1] Dillard, J., Dujon, V., & Brennan, E. M. (2013). 1. Introduction to Social Sustainability: A Multilevel Approach to Social Inclusion. In Social Sustainability: A Multilevel Approach to Social Inclusion (pp. 1–1). essay, Routledge. Retrieved January 21, 2025, from https://doi.org/10.4324/9780203751886..

[2] Franz, C. J. (2024). (thesis). PRODUCT DESIGN FOR REPAIRABILITY: IDENTIFYING FAILURE MODES WITH TOPIC MODELING AND DESIGNING FOR ELECTRONIC WASTE REUSE. Retrieved January 21, 2025, from https://www.proquest.com/docview/3122661181?%20Theses&pq-origsite=primo&sourcetype=Dissertations%20.

[3] Retolaza, I., Ezpeleta, I., Santos, A., Diaz, I., & Martinez, F. (n.d.). Design to cost; a framework for large industrial products. Procedia CIRP, 100, 828–883. https://doi.org/https://doi.org/10.1016/j.procir.2021.05.036

[4] Protolabs. (2023). Comparing Cost Between Injection Molding and 3D Printing. Retrieved from: https://www.protolabs.com/resources/blog/comparing-cost-between-injection-molding-and-3d-printing/

[5] "Why Empowering Independence Should Be Part of Your Elder Care Approach," Caregiving Kinetics, [Online]. Available: https://caregivingkinetics.com/why-empowering-independence-should-be-part-of-your-elder-care-approach/. [Accessed: 30-Jan-2025].

[6] A. P. George and J. T. F. L. Lee, "Obesity and Related Disorders," National Center for Biotechnology Information, [Online]. Available: https://www.ncbi.nlm.nih.gov/books/NBK310951/. [Accessed: 30-Jan-2025].

[7] J. W. Mark, "Obesity and Its Impact on the Health of Individuals," National Institutes of Health, [Online]. Available: https://pmc.ncbi.nlm.nih.gov/articles/PMC2768139/. [Accessed: 30-Jan-2025].

[8] J. S. P. Martin, "3D-Printed Prostheses: A Solution to Limb Loss in Low-Income Countries," Engineering for Change, [Online]. Available: https://www.engineeringforchange.org/news/3d-printed-prostheses-solution-limb-loss-low-income-countries/. [Accessed: 30-Jan-2025].

[9] Carbiolice. What is PLA?. Carbiolice. (2023, November 29). https://www.carbiolice.com/en/news/pla-bioplastic-2/

[10] Editor, P. P. C. (2024, February 29). New Report & Supplement Reveal the Hidden Costs of Top-Selling PLA Bioplastic. Plastic Pollution Coalition. https://www.plasticpollutioncoalition.org/blog/2024/2/29/the-hidden-costs-of-top-selling-pla  ‌